Multi-Use USB Temp&RH Data Logger

User Manual

1

Product Introduction

The device is mainly used to monitor the temperature and humidity of food, medicine, and other products during storage and transportation. After recording, insert it into the USB port of PC, it will automatically generate reports without any driver.

Main Features

- Multi-use temperature & humidity measuring and recording

- Widely measuring range, high accuracy and large data memory

- Statistics available on LCD screen

- No software needed to generate

- PDF and CSV report

Parameter programmable by configure software

2

Specification

| Item | Parameter |
| --- | --- |
| Temp Scale | ℃ or ℉ |
| Accuracy | ±3%RH;±0.5℃(-20℃ ~+40℃), ±1.0℃(other) |
| Range | -30℃ ~ 60℃;0%~85%RH |
| Resolution | 0.1 |
| Capacity | 32,000 readings |
| Startup Mode | Button or software |
| Interval | Optional Default: 10 mins |
| Start Delay | Optional Default: 30 mins |
| Alarm Delay | Optional Default: 10 mins |
| Alarm Range | Optional Default: <2℃ or >8℃, <40%RH or >80%RH |
| Shelf Life | 1 year (replaceable) |
| Report | Automatic PDF and CSV |
| Time Zone | UTC +0:00 (Default) |
| Dimension | 89mm*36mm*16mm |

3

How to use

- Start Recording

- Press and hold the “▶” button for more

than 3s until  the “ OK” light is on and the “▶” or “WAIT” displays on the screen, which indicates the logger is started.

- Mark

When the device is recording, press and hold the “▶” button for more than 3s, and the screen will switch to the “MARK” interface. The number of “MARK” will increase by one, indicating

data was marked successfully.

(Note: One record interval can mark one time only, the logger can mark 6 times in one recording trip. Under status of start delay, the mark operation                                                                                   is disabled.)

4

- Stop Recording

Press and hold the “■” button for more than 3s until the “ALARM” light is on, and

the “■” displays on the screen, indicating stopping recording successfully.

(Note: If the logger is stopped during the status of start delay, a PDF report generated when inserted into PC but without data.)

LCD Display Instruction

5

During normal recording process, shortly press “▶” to switch to different display interface. The interfaces shown in sequence are respectively:

Real-time Temperature Interface → Real-time Humidity Interface → LOG Interface → MARK Interface → Temperature Upper Limit Interface → Temperature Lower Limit Interface → Humidity Upper Limit Interface → Humidity Lower Limit Interface.

- Note:

- If the device is used for the first time or after re-configuration, the real-time temperature interface will be the initialization interface.

Real-time temperature and humidity        interface is update every 10s.

6

①Initialization (Temp)  ②Initialization(RH)

③Upper limit(Temp)   ④Lower limit(RH)

- ⑤Log interface       ⑥Mark interface

7

- ⑦Upper limit(Temp)  ⑧Lower limit(Temp)

- (Stop recording)       (Stop recording)

- ⑨Upper limit(RH)    ⑩Lower limit(RH)

- (Stop recording)       (Stop recording)

8

| ▶ | Data logger is recording |
| --- | --- |
| - ■ | Data logger has stopped recording |
| - WAIT | Data logger is in the status of start delay |
| - √ | Temperature and humidity is within the limited range |
| - “×” and “↑” light | Measured temperature and humidity exceeds its upper limit |
| - “×” and “↓” light | Temperature and humidity exceeds its lower limit |

9

- Battery Replacement

- Turn the battery cover counterclockwise to open it.

(Open status)

- Put in a new CR2032 button battery, with the negative inward.

- Turn the battery cover clockwise to close it.

(Closed status)

10

- Battery Status Indication

| Battery | Capacity |
| --- | --- |
|  | Full |
|  | Good |
|  | Medium |
|  | Low(please replace batery) |

- Precautions

- Please read the manual carefully before using the logger.

- It is recommended to check the battery status before restarting the logger to ensure that the remaining battery capacity could finish the recording task.

The LCD screen will be off after 10 seconds of inactivity. Please press the “▶” button to lighten it.

11

- Never dismantle the battery. Do not remove it if the logger is running.

- Replace old battery with a new CR2032 button cell with the negative inward.

12